RegPilot 合规审查报告

Regulation Compliance Analysis Report

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

一、基本信息

| 产品名称 | 折叠车 - 电助力车 |
| --- | --- |
| 产品类型 | 电助力车 |
| 审查日期 | 2026-06-03 13:36:48 |
| 报告编号 | RP-7296875C |
| 审查状态 | 已完成 |
| 法规体系 | CPSC 16 CFR 1512 / CPSIA / UL 2849 / UL 2271 |

二、合规概览

| 合规分数 64.6 | 风险分数 25.7 | 风险等级 MEDIUM | 合规状态 部分合规 |
| --- | --- | --- | --- |
| 合规: 17 | 部分合规: 6 | 待验证: 20 | 不合规: 0 |

三、适用法规

• CPSC 1512  (§1512.10(a)、§1512.10(b)、§1512.11(a)、§1512.12、§1512.13(a)、§1512.14(a)、§1512.15(a)、§1512.15(b)、§1512.16(a) & (c)、§1512.16(a) & (d)、§1512.16(b) & (f)、§1512.16(e) & §1512.7(c)、§1512.17(a) & §1512.18(p)、§1512.19(a)、§1512.19(a)(1)、§1512.19(b) & (c)、§1512.19(e)、§1512.2(b)(2)、§1512.4(a)、§1512.4(b)、§1512.5(a)、§1512.5(b)(1)、§1512.5(b)(3)、§1512.5(b)(5)、§1512.6(b) & §1512.18(g)、§1512.6(e)、§1512.7(a)、§1512.7(c)、§1512.9(b))

• State/Local  (e.g., NY VTL §12336-b)

• UL 2271  (Overcharge Test、Overdischarge Test、Short Circuit Test、Temperature Test)

• UL 2849  (Charger Requirements、Controller Requirements、Dielectric Withstand Test、Insulation Impedance Test、Motor Locked Rotor Test、Motor Overload Test、Software Evaluation & Functional Safety Assessment、Water Exposure Test - IPX4)

四、详细审查结果

| 序号 | 规则ID | 类别 | 要求摘要 | 法规来源 | 状态 | 风险 | 性质 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FRK-001 | Frame | Front fork must absorb ≥39.5J energy wit... | CPSC 1512 §1512.13(a) | 部分合规 | 严重 | 强制 |
| 2 | FRM-001 | Frame | Frame/fork: withstand 890N or 39.5J at f... | CPSC 1512 §1512.14(a) | 部分合规 | 严重 | 强制 |
| 3 | CHG-003 | Chain Guard | Derailleur must have guard to prevent ch... | CPSC 1512 §1512.9(b) | 部分合规 | 重要 | 强制 |
| 4 | WHL-001 | Wheel | Wheels must be securely attached; quick-... | CPSC 1512 §1512.11(a) | 部分合规 | 重要 | 强制 |
| 5 | HUB-001 | Wheel | Hub axles must be securely fastened and ... | CPSC 1512 §1512.12 | 部分合规 | 重要 | 强制 |
| 6 | TIR-002 | Wheel | Inflatable tires must have max pressure ... | CPSC 1512 §1512.10(b) | 部分合规 | 轻微 | 强制 |
| 7 | BRK-002 | Brake | Handbrake stopping distance ≤ 4.57m at 2... | CPSC 1512 §1512.5(b)(1) | 待验证 | 严重 | 强制 |
| 8 | BAT-001 | Battery | Battery must not fire/explode/smoke/leak... | UL 2271 Overcharge Test | 待验证 | 严重 | 强制 |
| 9 | BAT-002 | Battery | Battery must not fire/explode when short... | UL 2271 Short Circuit Test | 待验证 | 严重 | 强制 |
| 10 | BAT-003 | Battery | BMS must activate before hazardous state... | UL 2271 Overdischarge Test | 待验证 | 严重 | 强制 |
| 11 | BAT-004 | Battery | Battery temperatures must stay within UL... | UL 2271 Temperature Test | 待验证 | 严重 | 强制 |
| 12 | ELS-001 | Electrical Safety | E-bike electrical insulation resistance ... | UL 2849 Insulation Impedance Test | 待验证 | 严重 | 强制 |
| 13 | ELS-002 | Electrical Safety | E-bike must pass hi-pot test with no ins... | UL 2849 Dielectric Withstand Test | 待验证 | 严重 | 强制 |
| 14 | ELS-004 | Electrical Safety | BMS must pass software evaluation and fu... | UL 2849 Software Evaluation & Functional Safety Assessment | 待验证 | 严重 | 强制 |
| 15 | ELS-005 | Electrical Safety | Charger must meet UL 2849 safety require... | UL 2849 Charger Requirements | 待验证 | 严重 | 强制 |
| 16 | MOT-001 | Motor | Motor must not fire or create hazard und... | UL 2849 Motor Overload Test | 待验证 | 严重 | 强制 |
| 17 | MOT-002 | Motor | Motor must not fire or create hazard und... | UL 2849 Motor Locked Rotor Test | 待验证 | 严重 | 强制 |
| 18 | MOT-003 | Motor | Controller must have overcurrent and ove... | UL 2849 Controller Requirements | 待验证 | 严重 | 强制 |
| 19 | TST-005 | Testing | Stem: 2000N at 45° (1000N sidewalk) with... | CPSC 1512 §1512.6(b) & §1512.18(g) | 待验证 | 严重 | 强制 |
| 20 | BRK-003 | Brake | Handbrake grip dimension ≤ 89mm (76mm si... | CPSC 1512 §1512.5(b)(3) | 待验证 | 重要 | 强制 |
| 21 | BRK-004 | Brake | Handbrake operating force < 44.5N at 25m... | CPSC 1512 §1512.5(b)(5) | 待验证 | 重要 | 强制 |
| 22 | ELS-003 | Electrical Safety | E-bike must pass IPX4 splash-water test ... | UL 2849 Water Exposure Test - IPX4 | 待验证 | 重要 | 强制 |
| 23 | TST-003 | Testing | Non-sidewalk bikes must pass 6.4km road ... | CPSC 1512 §1512.17(a) & §1512.18(p) | 待验证 | 重要 | 强制 |
| 24 | MEC-001 | Mechanical | No exposed sharp edges on any user-conta... | CPSC 1512 §1512.4(a) | 待验证 | 重要 | 强制 |
| 25 | LBL-003 | Label | Seat post must have permanent min-insert... | CPSC 1512 §1512.15(b) | 待验证 | 重要 | 强制 |
| 26 | LBL-005 | Label | Unassembled bikes must show required too... | CPSC 1512 §1512.19(b) & (c) | 待验证 | 轻微 | 强制 |
| 27 | REF-001 | Reflector | Must have essentially colorless front re... | CPSC 1512 §1512.16(a) & (c) | 合规 | 重要 | 强制 |
| 28 | REF-002 | Reflector | Must have red rear reflector on seat pos... | CPSC 1512 §1512.16(a) & (d) | 合规 | 重要 | 强制 |
| 29 | REF-003 | Reflector | Pedals must have colorless or amber refl... | CPSC 1512 §1512.16(e) & §1512.7(c) | 合规 | 轻微 | 强制 |
| 30 | REF-004 | Reflector | Wheels must have side reflectors (tire/s... | CPSC 1512 §1512.16(b) & (f) | 合规 | 轻微 | 强制 |
| 31 | BEL-001 | Bell | Bell or audible warning device recommend... | State/Local e.g., NY VTL §12336-b | 合规 | 轻微 | 建议 |
| 32 | BRK-001 | Brake | Must have front and rear brakes or rear-... | CPSC 1512 §1512.5(a) | 合规 | 严重 | 强制 |
| 33 | TIR-001 | Wheel | Tires must stay on rim at max rated pres... | CPSC 1512 §1512.10(a) | 合规 | 重要 | 强制 |
| 34 | MEC-002 | Mechanical | No protrusions; handlebar caps ≥66.8N; c... | CPSC 1512 §1512.4(b) | 合规 | 重要 | 强制 |
| 35 | MOT-004 | Motor | E-bike: motor <750W; Class 1/2 ≤20mph, C... | CPSC 1512 §1512.2(b)(2) | 合规 | 严重 | 强制 |
| 36 | WRN-002 | Warning | Manual must warn about over-tightening s... | CPSC 1512 §1512.6(e) | 合规 | 重要 | 强制 |
| 37 | WRN-003 | Warning | Manual must include night riding safety ... | CPSC 1512 §1512.19(a)(1) | 合规 | 轻微 | 强制 |
| 38 | WRN-004 | Warning | Manual must include wet weather operatio... | CPSC 1512 §1512.19(a)(1) | 合规 | 轻微 | 强制 |
| 39 | FRM-002 | Frame | Seat post clamp must hold seat post secu... | CPSC 1512 §1512.15(a) | 合规 | 重要 | 强制 |
| 40 | LBL-001 | Label | Frame must have permanent label with man... | CPSC 1512 §1512.19(e) | 合规 | 重要 | 强制 |
| 41 | LBL-004 | Label | Must include instruction manual with ope... | CPSC 1512 §1512.19(a) | 合规 | 重要 | 强制 |
| 42 | PED-001 | Drivetrain | Pedals must have anti-slip tread on both... | CPSC 1512 §1512.7(a) | 合规 | 轻微 | 强制 |
| 43 | PED-002 | Drivetrain | Pedal reflectors must be colorless or am... | CPSC 1512 §1512.7(c) | 合规 | 轻微 | 强制 |

五、不合规项与部分合规项

1. [FRK-001] Front fork must absorb ≥39.5J energy with ≤64mm deflection; no fractures

法规来源：CPSC 1512 §1512.13(a)

状态：部分合规 | 风险：严重 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Ensure front fork passes static load test at 400N without permanent deformation per §1512.13(a).

后果：Recall；CPSC Fine；Lawsuit

2. [FRM-001] Frame/fork: withstand 890N or 39.5J at fork axle; no fracture or steering limit

法规来源：CPSC 1512 §1512.14(a)

状态：部分合规 | 风险：严重 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Ensure frame and fork assembly passes road test without structural failure per §1512.14(a).

后果：Recall；CPSC Fine；Lawsuit

3. [CHG-003] Derailleur must have guard to prevent chain from blocking wheel rotation

法规来源：CPSC 1512 §1512.9(b)

状态：部分合规 | 风险：重要 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Install a derailleur guard that prevents chain from interfering with or stopping wheel rotation per §1512.9(b).

后果：Customs Hold；Lawsuit

4. [WHL-001] Wheels must be securely attached; quick-release must have positive locking and open/closed markings

法规来源：CPSC 1512 §1512.11(a)

状态：部分合规 | 风险：重要 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Ensure wheels are securely attached and quick-release mechanisms have positive locking with open/closed markings per §1512.11(a).

后果：Recall；Customs Hold；Lawsuit

5. [HUB-001] Hub axles must be securely fastened and not loosen during normal use

法规来源：CPSC 1512 §1512.12

状态：部分合规 | 风险：重要 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Ensure hub axles are properly tightened to manufacturer's specified torque per §1512.12.

后果：Customs Hold；Lawsuit

6. [TIR-002] Inflatable tires must have max pressure rating marked on sidewall

法规来源：CPSC 1512 §1512.10(b)

状态：部分合规 | 风险：轻微 | 性质：强制

差距描述：BOM确认部分组件存在，但存在缺失项，且部分测试待验证

整改建议：Mark maximum inflation pressure on tire sidewall per §1512.10(b).

后果：Customs Hold

六、待验证项

1. [FRK-001] Front fork must absorb ≥39.5J energy with ≤64mm deflection; no fractures

需提供的验证材料：测试报告：Front fork static load test report per §1512.13(a) showing no permanent deformation at 400N

2. [FRM-001] Frame/fork: withstand 890N or 39.5J at fork axle; no fracture or steering limit

需提供的验证材料：测试报告：Frame assembly road test report per §1512.18(p) showing no structural failure

3. [CHG-003] Derailleur must have guard to prevent chain from blocking wheel rotation

需提供的验证材料：产品照片：Product photo showing derailleur guard installed

4. [WHL-001] Wheels must be securely attached; quick-release must have positive locking and open/closed markings

需提供的验证材料：产品照片：Product photo showing quick-release mechanism with open/closed position markings

5. [HUB-001] Hub axles must be securely fastened and not loosen during normal use

需提供的验证材料：规格参数：Spec sheet documenting hub axle torque specifications

6. [TIR-002] Inflatable tires must have max pressure rating marked on sidewall

需提供的验证材料：产品照片：Product photo showing tire sidewall pressure marking

7. [BRK-002] Handbrake stopping distance ≤ 4.57m at 24km/h (or 16km/h if max speed <24km/h), lever force ≤178N

需提供的验证材料：测试报告：Third-party lab report showing braking distance test results per §1512.18(d)(2)(vi)

8. [BAT-001] Battery must not fire/explode/smoke/leak when overcharged to 110% max voltage under single fault

需提供的验证材料：测试报告：UL 2271 overcharge test report showing no fire, explosion, smoke, or leakage under single fault condition；规格参数：BMS protection circuit verification documentation

9. [BAT-002] Battery must not fire/explode when short-circuited (≤20mΩ) under single fault

需提供的验证材料：测试报告：UL 2271 short circuit test report showing no fire or explosion

10. [BAT-003] BMS must activate before hazardous state during overdischarge under single fault

需提供的验证材料：测试报告：UL 2271 overdischarge test report showing BMS activation before hazardous state；规格参数：BMS overdischarge protection verification documentation

11. [BAT-004] Battery temperatures must stay within UL 2271 Table 26.1/26.2 limits over 5 cycles at max temp

需提供的验证材料：测试报告：UL 2271 temperature test report with all component and surface temperature records over 5 cycles；规格参数：Temperature records for all components and accessible surfaces

12. [ELS-001] E-bike electrical insulation resistance must exceed specified minimums

需提供的验证材料：测试报告：UL 2849 insulation impedance test report showing resistance above minimum values

13. [ELS-002] E-bike must pass hi-pot test with no insulation breakdown at elevated voltage

需提供的验证材料：测试报告：UL 2849 dielectric withstand (hi-pot) test report showing no insulation breakdown

14. [ELS-004] BMS must pass software evaluation and functional safety assessment

需提供的验证材料：测试报告：UL 2849 software evaluation report for BMS；规格参数：Functional safety assessment documentation for BMS software

15. [ELS-005] Charger must meet UL 2849 safety requirements (input, temp, protection)

需提供的验证材料：测试报告：UL 2849 charger test report including input, temperature, and protection function tests；规格参数：Charger specification sheet documenting safety features and protection functions

16. [MOT-001] Motor must not fire or create hazard under overload conditions

需提供的验证材料：测试报告：UL 2849 motor overload test report showing no fire or safety hazard

17. [MOT-002] Motor must not fire or create hazard under locked rotor conditions

需提供的验证材料：测试报告：UL 2849 motor locked rotor test report showing no fire or safety hazard

18. [MOT-003] Controller must have overcurrent and overtemperature auto-protection

需提供的验证材料：规格参数：Controller specification sheet documenting overcurrent and overtemperature protection functions；测试报告：UL 2849 component fault test report verifying controller protection functions

19. [TST-005] Stem: 2000N at 45° (1000N sidewalk) without fracture

需提供的验证材料：测试报告：Handlebar stem strength test report per §1512.6(b) & §1512.18(g)

20. [BRK-003] Handbrake grip dimension ≤ 89mm (76mm sidewalk); max increase 12.7mm toward open end

需提供的验证材料：规格参数：Spec sheet or drawing showing handbrake grip dimension measurement

21. [BRK-004] Handbrake operating force < 44.5N at 25mm from open end

需提供的验证材料：测试报告：Test report showing handbrake operating force measurement

22. [ELS-003] E-bike must pass IPX4 splash-water test without safety hazard

需提供的验证材料：测试报告：UL 2849 IPX4 water exposure test report showing no safety hazard after splash testing

23. [TST-003] Non-sidewalk bikes must pass 6.4km road test with 68.1kg rider and 5 bump passes

需提供的验证材料：测试报告：Road test report per §1512.17(a) & §1512.18(p) with test conditions and results

24. [MEC-001] No exposed sharp edges on any user-contactable parts

需提供的验证材料：测试报告：Sharp edge test report per 16 CFR 1500.49 showing no accessible sharp edges；产品照片：Product photos showing all edges are smoothed or rolled

25. [LBL-003] Seat post must have permanent min-insertion mark ≥2.5× diameter from bottom

需提供的验证材料：规格参数：Spec sheet or drawing noting seat post minimum insertion depth mark；产品照片：Product photo showing insertion depth mark on seat post

26. [LBL-005] Unassembled bikes must show required tools and min leg length on packaging

需提供的验证材料：包装照片：Shipping carton photo showing required tools list and minimum leg length diagram；包装照片：Promotional display material sample showing required markings

27. [REF-001] Must have essentially colorless front reflector

需提供的验证材料：测试报告：Reflector performance test report per Table 1 & 2 of §1512.16

28. [REF-002] Must have red rear reflector on seat post, top ≥76mm below seat

需提供的验证材料：规格参数：Spec sheet must confirm rear reflector mounting position relative to seat post；测试报告：Reflector performance test report per §1512.16(d)

29. [REF-003] Pedals must have colorless or amber reflectors on front and rear

需提供的验证材料：测试报告：Pedal reflector performance test report per §1512.16(e)

30. [REF-004] Wheels must have side reflectors (tire/spoke/rim); front: colorless/amber, rear: colorless/red

需提供的验证材料：测试报告：Side reflector performance test report per §1512.16(f)

31. [TIR-001] Tires must stay on rim at max rated pressure for 2 hours without separation

需提供的验证材料：测试报告：Tire retention test report per §1512.10(a) showing no tire-rim separation at max rated pressure

32. [MEC-002] No protrusions; handlebar caps ≥66.8N; cable ends capped; cables no abrasion

需提供的验证材料：产品照片：Product photo showing handlebar ends properly capped and cables properly routed

33. [MOT-004] E-bike: motor <750W; Class 1/2 ≤20mph, Class 3 ≤28mph; must have operable pedals

需提供的验证材料：测试报告：Maximum speed test report with 170 lb rider on paved level surface under motor power only

34. [FRM-002] Seat post clamp must hold seat post securely without slipping

需提供的验证材料：测试报告：Seat post clamp holding test report per §1512.15(a) showing no slippage

35. [PED-001] Pedals must have anti-slip tread on both sides or self-right, no sharp edges

需提供的验证材料：产品照片：Product photo showing pedal tread surfaces on both sides or self-righting design

36. [PED-002] Pedal reflectors must be colorless or amber, meeting §1512.16(e) photometric requirements

需提供的验证材料：测试报告：Pedal reflector photometric test report per §1512.16(e)

七、产品组件清单

| 序号 | 原始名称 | 标准名称 | 类别 | 规格 | 数量 |
| --- | --- | --- | --- | --- | --- |
| 1 | Wal-mart US Concord 20"23CF2001YD E-Folding Bike | unknown | Unknown |  | 1 |
| 2 | 烤漆车架 | frame_main | Frame |  | 1 |
| 3 | 车架 | frame_main | Frame |  | 1 |
| 4 | 贴花 | label | Label |  | 1 |
| 5 | 烤漆前叉 | fork_front | Frame |  | 1 |
| 6 | 前叉 | fork_front | Frame |  | 1 |
| 7 | 车把 | handlebar | Steering |  | 1 |
| 8 | 立管 | handlebar_stem | Steering |  | 1 |
| 9 | 把套 | handlebar_grip | Steering |  | 1 |
| 10 | 车把尾塞 | handlebar_grip | Steering |  | 1 |
| 11 | 前叉组件 | headset | Steering |  | 1 |
| 12 | 断电刹把 | ebrake_lever | Brake |  | 1 |
| 13 | 前刹车器 | handbrake_front | Brake |  | 1 |
| 14 | 后刹车器 | handbrake_rear | Brake |  | 1 |
| 15 | 前刹车线 | brake_cable | Brake |  | 1 |
| 16 | 后刹车线 | brake_cable | Brake |  | 1 |
| 17 | 前轮轮组 | wheel_front | Wheel |  | 1 |
| 18 | 前花鼓 | hub_front | Wheel |  | 1 |
| 19 | 车圈 | rim | Wheel |  | 1 |
| 20 | 辐条 | spoke | Wheel |  | 1 |
| 21 | 辐条 | spoke | Wheel |  | 1 |
| 22 | 外胎 | tire_front | Wheel |  | 1 |
| 23 | 内胎 | inner_tube | Wheel |  | 1 |
| 24 | 衬带 | rim_tape | Wheel |  | 1 |
| 25 | 碟刹盘 | brake_disc | Brake |  | 1 |
| 26 | 后轮轮组 | wheel_rear | Wheel |  | 1 |
| 27 | 轮毂电机 | motor | Electrical |  | 1 |
| 28 | 车圈 | rim | Wheel |  | 1 |
| 29 | 辐条 | spoke | Wheel |  | 1 |
| 30 | 辐条 | spoke | Wheel |  | 1 |
| 31 | 外胎 | tire_front | Wheel |  | 1 |
| 32 | 内胎 | inner_tube | Wheel |  | 1 |
| 33 | 衬带 | rim_tape | Wheel |  | 1 |
| 34 | 碟刹盘 | brake_disc | Brake |  | 1 |
| 35 | 飞轮 | freewheel | Drivetrain |  | 1 |
| 36 | 链条 | chain | Drivetrain |  | 1 |
| 37 | 脚踏 | pedal | Drivetrain |  | 1 |
| 38 | 中轴 | bottom_bracket | Drivetrain |  | 1 |
| 39 | 链轮曲柄 | crankset | Drivetrain |  | 1 |
| 40 | 后拨链器 | derailleur_rear | Drivetrain |  | 1 |
| 41 | 右变速手柄 | shifter | Drivetrain |  | 1 |
| 42 | 变速线管 | shifter | Drivetrain |  | 1 |
| 43 | 停车架 | kickstand | Accessory |  | 1 |
| 44 | 电子转把 | throttle | Electrical |  | 1 |
| 45 | 前灯 | light_front | Electrical |  | 1 |
| 46 | 锂电池包 | battery_pack | Electrical |  | 1 |
| 47 | 电池盒 | battery_pack | Electrical |  | 1 |
| 48 | 充电器 | charger | Electrical |  | 1 |
| 49 | 速度传感器 | speed_sensor | Electrical |  | 1 |
| 50 | 仪表 | display | Electrical |  | 1 |
| 51 | 控制器 | motor_controller | Electrical |  | 1 |
| 52 | 防水线 | wire_harness | Electrical |  | 1 |
| 53 | 前泥板 | fender | Accessory |  | 1 |
| 54 | 后泥板 | fender | Accessory |  | 1 |
| 55 | 前泥板支撑 | mudguard_stay | Accessory |  | 1 |
| 56 | 后泥板支撑 | mudguard_stay | Accessory |  | 1 |
| 57 | 鞍座 | seat | Frame |  | 1 |
| 58 | 鞍管 | seat_post | Frame |  | 1 |
| 59 | 鞍管快拆 | seat_clamp | Frame |  | 1 |
| 60 | 货架 | rack | Accessory |  | 1 |
| 61 | 轮反光片 | reflector_spoke | Reflector |  | 1 |
| 62 | 前反光片 | reflector_front | Reflector |  | 1 |
| 63 | 前反光片架 | reflector_front | Reflector |  | 1 |
| 64 | 后反光片 | reflector_rear | Reflector |  | 1 |
| 65 | 车铃 | bell | Bell |  | 1 |
| 66 | 标准件 | unknown | Unknown |  | 1 |
| 67 | 标准件 | unknown | Unknown |  | 1 |
| 68 | 标准件 | unknown | Unknown |  | 1 |
| 69 | 标准件 | unknown | Unknown |  | 1 |
| 70 | 标准件 | unknown | Unknown |  | 1 |
| 71 | 标准件 | unknown | Unknown |  | 1 |
| 72 | 标准件 | unknown | Unknown |  | 1 |
| 73 | 标准件 | unknown | Unknown |  | 1 |
| 74 | 标准件 | unknown | Unknown |  | 1 |
| 75 | 标准件 | unknown | Unknown |  | 1 |
| 76 | 标准件 | unknown | Unknown |  | 1 |
| 77 | 标准件 | unknown | Unknown |  | 1 |
| 78 | 工具 | tool_kit | Accessory |  | 1 |
| 79 | 线卡 | unknown | Unknown |  | 1 |
| 80 | 束口袋 | label | Label |  | 1 |
| 81 | 吊卡 | label | Label |  | 1 |
| 82 | 轮卡 | label | Label |  | 1 |
| 83 | 吊卡 | label | Label |  | 1 |
| 84 | 说明书 | manual | Label |  | 1 |
| 85 | 充电器防撕标纸 | warning_label | Label |  | 1 |
| 86 | 防退货小标识 | label | Label |  | 1 |
| 87 | 不干胶标签 | label | Label |  | 1 |
| 88 | 不干胶标签 | label | Label |  | 1 |
| 89 | 不干胶标签 | label | Label |  | 1 |
| 90 | 不干胶标签 | label | Label |  | 1 |
| 91 | 不干胶标签 | label | Label |  | 1 |
| 92 | 不干胶标签 | label | Label |  | 1 |
| 93 | 前叉防撞贴 | unknown | Unknown |  | 1 |
| 94 | 防撞贴 | unknown | Unknown |  | 1 |
| 95 | 缠绕管 | unknown | Unknown |  | 1 |
| 96 | 易拉得 | unknown | Unknown |  | 1 |
| 97 | 易拉得 | unknown | Unknown |  | 1 |
| 98 | 纸箱 | label | Label |  | 1 |
| 99 | 包装辅材 | label | Label |  | 1 |

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

免责声明：本报告由 RegPilot 系统基于上传的 BOM 数据自动生成，仅供参考。实际合规状态应以第三方实验室测试报告和法规原文为准。

Generated by RegPilot v1.0 | 2026-06-03 13:36